# Robot Eel Straight and Turning Gait Parameters

Purpose. This document summarizes the currently used fixed straight, turning, and spin gaits for the MuJoCo robot eel model. The gait generator is a Hopf CPG with per-joint amplitude targets represented internally by mu_i, phase lag, and steering bias.

# 1. Hopf CPG Used in the Simulation

For joint i, the controller uses the following oscillator form:

r_dot_i = alpha (mu_i - r_i^2) r_i + k_fb_amp fb_amp

theta_dot_i = omega + coupling_i + anchor_i + k_fb_phase fb_phase

mu_i = amp_i^2

q_i = A_joint r_i cos(theta_i) + b_i

where q_i is the target joint angle, amp_i is the human-readable per-joint amplitude scale, mu_i is the Hopf amplitude target, b_i is the joint steering bias, and omega = 2 pi f.

| Parameter | Current value | Meaning |
| --- | --- | --- |
| A_joint | 0.45 rad | Base joint amplitude |
| frequency | 1.0 Hz | Oscillation frequency |
| wavelength | 1.6275 | Nominal body wave parameter |
| phase lag | 0.614439 rad/joint | Phase difference between adjacent joints |
| alpha | 4.00 | Hopf amplitude convergence speed |
| k_couple | 0.35 | Adjacent-joint phase synchronization gain |
| k_anchor | 0.10 | Reference wave anchoring gain |

# 2. Fixed Gait Parameter Table

| Gait | File | A_joint | f (Hz) | lambda | amp_scales | phase_lags | joint_bias |
| --- | --- | --- | --- | --- | --- | --- | --- |
| straight | straight.json | 0.450 | 1.000 | 1.6275 | 1.240, 1.080, 1.000, 1.050, 1.100, 1.200 | 0.614439, 0.614439, 0.614439, 0.614439, 0.614439 | 0.000, 0.000, 0.000, 0.000, 0.000, 0.000 |
| turn_left | turn_left.json | 0.450 | 1.000 | 1.6275 | 1.240, 1.080, 1.000, 1.050, 1.100, 1.200 | 0.614439, 0.614439, 0.614439, 0.614439, 0.614439 | 0.080, 0.100, 0.120, 0.140, 0.160, 0.180 |
| turn_right | turn_right.json | 0.450 | 1.000 | 1.6275 | 1.240, 1.080, 1.000, 1.050, 1.100, 1.200 | 0.614439, 0.614439, 0.614439, 0.614439, 0.614439 | -0.080, -0.100, -0.120, -0.140, -0.160, -0.180 |
| spin_left | spin_left.json | 0.450 | 1.000 | 1.6275 | 1.240, 1.080, 1.000, 1.050, 1.100, 1.200 | 0.614439, 0.614439, 0.614439, 0.614439, 0.614439 | 0.120, 0.150, 0.180, 0.210, 0.240, 0.270 |
| spin_right | spin_right.json | 0.450 | 1.000 | 1.6275 | 1.240, 1.080, 1.000, 1.050, 1.100, 1.200 | 0.614439, 0.614439, 0.614439, 0.614439, 0.614439 | -0.120, -0.150, -0.180, -0.210, -0.240, -0.270 |

# 3. Straight Swimming Gait

The straight gait uses zero joint bias, so the body wave is symmetric and the eel mainly generates forward thrust. The current straight-swim amplitude pattern is:

1.240, 1.080, 1.000, 1.050, 1.100, 1.200

This pattern already increases the head-side and tail-side amplitudes relative to the middle joints. Therefore, the tail amplitude is already larger during straight swimming.

# 4. Turning and Spin Gaits

Turning is produced by adding a static joint_bias to the CPG output. Positive bias bends the body to one side, while negative bias mirrors the gait in the opposite direction. The phase lags are kept fixed for these saved gaits.

| Gait | joint_bias |
| --- | --- |
| turn_left | 0.080, 0.100, 0.120, 0.140, 0.160, 0.180 |
| turn_right | -0.080, -0.100, -0.120, -0.140, -0.160, -0.180 |
| spin_left | 0.120, 0.150, 0.180, 0.210, 0.240, 0.270 |
| spin_right | -0.120, -0.150, -0.180, -0.210, -0.240, -0.270 |

# 5. Rectangle Course Controller

The rectangle-course viewer does not directly switch between the saved fixed gaits. Instead, it computes a steering command from the path-following controller, smooths it, then converts it into joint_bias and a tail-amplitude modulation.

steer_state <- steer_state + beta (target_steer - steer_state)

joint_bias_i = w_i steer_state

w = [0.45, 0.55, 0.68, 0.80, 0.92, 1.00]

The default smoothing beta is 0.08. Smaller beta gives smoother but slower turning response.

| Case | amp_scales | mu_scales |
| --- | --- | --- |
| Straight/base | 1.240, 1.080, 1.000, 1.050, 1.100, 1.200 | 1.538, 1.166, 1.000, 1.103, 1.210, 1.440 |
| Turning example steer=0.18, gain=0.6 | 1.240, 1.080, 1.016, 1.090, 1.177, 1.330 | 1.538, 1.166, 1.033, 1.187, 1.386, 1.768 |

The turning amplitude modulation is applied through Hopf mu_i targets. Because r_i is a Hopf state, the amplitude moves toward the new value smoothly instead of jumping instantly.

# 6. Practical Notes

Direct sine control is simple for a fixed gait, but changing wavelength or amplitude during swimming can create abrupt target-angle changes.

Hopf CPG keeps internal r_i and theta_i states, so online gait changes are continuous and smoother.

For the current model, phase lag is fixed during rectangle turning; turning mainly uses joint_bias plus tail amplitude modulation.

The current slower Hopf gains make gait transitions smoother, but the gait takes longer to converge after startup or after a turn command changes.